ISSSP 2026 — TRACK 3: LUNAR SURFACE ROBOTICS · FINALS SCORING CRITERION 2
Official Competition Poster Specification & Blueprint

Complete Scientific Poster Copy, Technical Schemes, 3-Column Architectural Layout, and Artistry Design Guidelines (Deadline: 31st Oct — Max Score: 10/10)

| Poster Title: L-MINER Mk-II: Autonomous Multi-Tier Lunar Prospector | Format & Dims: ISO A0 Portrait (841 x 1189 mm) / 300 DPI |
| --- | --- |
| Scoring Focus: Table 6: Item 2 (Project Scheme, Background, Methods) | Author Roster: Haider Taqvee, Abdul Wasay, Saad, Hajra (IST Islamabad) |

# 1. Poster Design Specifications & Scoring Mandate

According to Table 6 (Finals Scoring Rules of Track 3), the Poster deliverable (due 31st October) carries 10 points. The evaluators score the poster on: 'Project scheme display, Include Team Information, Background Introduction, Research Content And Methods, etc.' Furthermore, it directly influences the 5-point 'Artistry' score in the Finals.

Physical Dimensions: Standard ISO A0 Portrait (841 mm width x 1189 mm height; 33.1 x 46.8 inches).

Digital Canvas: 9933 x 14043 pixels at 300 DPI (CMYK Color Space for professional high-contrast printing).

Grid System: 3-Column Symmetric Grid with 30 mm margins and 20 mm inter-column gutters.

Visual Flow: Top Header -> Top-to-Bottom Column 1 (Context & Autonomy) -> Column 2 (Sensors & ML) -> Column 3 (Simulation HUD & ISRU Yield).

Judge Scoring Alignment Poster Evaluation Strategy: Judges evaluate scientific clarity, novelty of the technical scheme, and aesthetic readability from a distance of 1.5 to 2.0 meters. Bullet points are concise, diagrams are high-resolution, and equations are explicitly annotated.

# 2. Poster Header & Team Information Block

Header Panel Copy (Exact Print Text) POSTER TITLE (96 pt Bold): L-MINER Mk-II: Autonomous Multi-Tier Rover for Subsurface Lunar Volatile Prospecting and Mineral Classification SUBTITLE (44 pt Medium): Dual-Modality Subsurface GPR and VNIR-SWIR Spectrometry with Edge Machine Learning for Lunar South Pole Cold Traps AUTHOR LIST (32 pt Semi-Bold): Haider Taqvee¹, Abdul Wasay¹, Saad¹, Hajra¹ AFFILIATION (24 pt Regular): ¹ Department of Space Science & Aeronautics, Institute of Space Technology (IST), Islamabad, Pakistan ISSSP 2026 — Track 3: Lunar Surface Robotics and ISRU Exploration ¦ Project ID: LMINER-IST-2026

# 3. Print-Ready Text Copy Across All Poster Panels

## Panel 1: Background Introduction & Scientific Problem

The Lunar South Pole features Permanently Shadowed Regions (PSRs) where temperatures remain below 40 Kelvin. Orbital datasets (Chandrayaan-1 M³, LRO LAMP) confirm surface frost, but cannot resolve subsurface stratigraphy or volume.

The Ground-Truth Gap: Optical cameras are blinded by darkness and only sample the top 10 microns of dust. Earth teleoperation is impossible due to communication shadows and 3-second roundtrip delays.

Core Mission Objective: Develop an autonomous rover combining subsurface radar sounding (0–3 m) with active optical spectroscopy (0.5–2.5 μm) and edge machine learning to map water ice and titanium-rich minerals in real time.

## Panel 2: Hierarchical Autonomy Architecture (L0 / L1 / L2)

L-MINER implements a 3-tier decoupled autonomy framework guaranteeing mission safety and battery preservation:

• L0 Safety Reflex (10 Hz): Real-time obstacle bypass using HazCam point-clouds. Blocks drive into slopes >15° or boulders >0.75 hazard score.

• L1 Resource & Thermal Manager (1 Hz): Monitors battery state of charge (SoC) and electronics temperatures. Automatically pauses scans at SoC < 30% or T > 50°C and initiates solar panel recharge cycles.

• L2 Science Prospecting (On Demand): Executes multi-physics sensor sounding, edge feature engineering, and Random Forest classification upon reaching designated science coordinates.

## Panel 3: Multi-Physics Sensor Simulation & Physical Modeling

Synthetic sensor modeling grounded in Apollo 11/12 sample returns and USGS splib07 spectral libraries:

• Ground Penetrating Radar (500 MHz): Synthesizes 128 x 256 B-scan profiles across 0–3.0 m depth. Reflects via dielectric permittivity contrast: Water Ice (A=0.85, width 3), Pyroxene (A=0.65), Ilmenite (A=0.55), Regolith (A=0.30).

• VNIR-SWIR Spectrometer (0.5–2.5 μm, 200 channels): Active illuminator enables sensing inside dark craters. Captures diagnostic absorption bands: Water Ice (1.25, 1.5, 2.0 μm), Pyroxene (1.0 & 2.0 μm Fe²⁺ crystal field bands), Ilmenite (Fe-Ti charge transfer flattening, R ~ 0.07), and space-weathered regolith red slope.

## Panel 4: Edge Feature Extraction & Machine Learning Classification

Edge Data Reduction: Reduces 32,968 raw floats into a 10-dimensional feature vector (3,300x compression):
  [0–3] GPR Features: Max amplitude, mean energy, layer standard deviation, peak echo depth.
  [4–9] Spectral Features: Mean albedo, R(1.0 μm), R(1.5 μm), R(2.0 μm), spectral slope, 1.5 μm band depth.

Random Forest Classifier (100 Trees, Gini Impurity):
• 95.8% Overall Multiclass Test Accuracy (Water Ice: 99.2%, Ilmenite: 96.5%, Pyroxene: 94.8%, Regolith: 92.5%).
• Sub-15 ms Inference Latency on radiation-tolerant ARM Cortex-R5 / RTG4 FPGA embedded processors.

## Panel 5: Interactive 3D Digital Twin & Mission Control HUD

Full cyber-physical demonstration environment built with Streamlit and Plotly WebGL:

• 3D Lunar Polar Digital Twin: High-density 96 x 96 crater terrain mesh with 3° low-angle polar solar illumination, 6-wheel rocker-bogie rover mesh, terrain-following pitch/roll adaptation, and subsurface deposit spheres sized by ML confidence.

• Aerospace Mission HUD: Real-time telemetry cards, dual GPR B-scan + A-scan energy plots, USGS laboratory benchmark overlay curves, and autonomous survey sequence controls.

## Panel 6: Quantitative ISRU Yields & Mission Impact

In-Situ Resource Utilization (ISRU) Potential:
• In a 6 x 6 grid survey (60 x 60 m zone), 7 water ice deposits were localized at average 0.9 m depth.
• Yield Calculation: 7 cells x (100 m² x 1.0 m thickness x 1500 kg/m³ density x 5.5 wt% ice) = ~57.8 Tonnes H₂O.
• Enables production of ~51.4 Tonnes of Liquid Oxygen (LOX) and ~6.4 Tonnes of Liquid Hydrogen (LH2) propellant, sufficient to refuel two lunar ascent vehicles for return to Earth orbit.

# 4. Poster Layout Wireframe & Graphic Design Blueprint

| Column 1 (Width: 247 mm) | Column 2 (Width: 247 mm) | Column 3 (Width: 247 mm) |
| --- | --- | --- |
| SECTION 1: MISSION DRIVERS • Lunar South Pole Cold Traps • Ground Truth Gap diagram • High-contrast crater photo | SECTION 3: SENSORS & PHYSICS • 500 MHz GPR underbelly diagram • VNIR-SWIR Spectrometer arm • Dielectric vs Reflectance curves | SECTION 5: 3D DIGITAL TWIN • 3D Rover mesh on polar terrain • Subterranean deposit ellipsoids • GPR sounding radar cone render |
| SECTION 2: HIERARCHICAL AUTONOMY • L0 Safety Reflex state diagram • L1 Battery & Thermal thresholds • L2 Science trigger logic | SECTION 4: MACHINE LEARNING • 10-D Feature extraction flowchart • Random Forest architecture • Confusion Matrix (95.8% Acc) | SECTION 6: ISRU VOLATILE MAP • 6x6 Survey grid resource map • ~57.8 Tonnes Water Ice calculation • LOX/LH2 Propellant production chart |
| FIGURE 1: Rover mechanical layout & sensor integration callouts. | FIGURE 2: GPR B-scan paired with A-scan energy profile & USGS comparison. | FIGURE 3: Live Streamlit Mission HUD screenshot with real-time telemetry. |
| PANEL FOOTER 1: Funding & IST Department Acknowledgments. | PANEL FOOTER 2: Key References (Apollo, USGS splib07, Chandrayaan-1 M³). | PANEL FOOTER 3: QR Code linking to live interactive simulation. |

# 5. Color Palette & Typography Specifications (Artistry - 5 Pts)

| Color Role | Hex Code | RGB Values | Application on Poster |
| --- | --- | --- | --- |
| Deep Space Black | #030712 | RGB(3, 7, 18) | Poster background, header background, terminal blocks |
| Cyan Telemetry Accent | #00E5FF | RGB(0, 229, 255) | Water ice features, radar cones, section titles, key metrics |
| Safety Green | #00E676 | RGB(0, 230, 118) | L0 nominal status, Pyroxene mineral indicators, positive callouts |
| Titanium Amber | #FF9100 | RGB(255, 145, 0) | Ilmenite signatures, L1 power warnings, dielectric reflectors |
| Hazard Red | #FF1744 | RGB(255, 23, 68) | Obstacle pillars, collision warning zones, low-power alerts |